Keystone Library

CONNOR LANGSFORD

Keystone Library

Connor Langsford

2/21/25

Abstract

Design documents of the creation of a Library in Noblesville, IN

Table of Contents

| Description: | Page Number(s) |
| --- | --- |
| Project Description | 2 |
| Building Code and Regulations | 3-5 |
| Composite Floor Load-Span table | 6 |
| Roof Load | 7 |
| Beam Calculations | 8-19 |
| Spread Footing | 20-21 |
| Wastewater Discharge | 22 |
| Framing | 23 |
| First Floor Plan | 24 |
| Second Floor Plan | 25 |
| Elevation Views | 26 |
| Section Views | 27 |
| Landscaping | 28 |
| Site Plan | 29 |
| Schedules | 30 |

Project Description

-Overview

Design of the redevelopment of an existing industrial brownfield site in Noblesville, IN.

-Criteria and Constraints

- A safe and efficient building with adequate utilities and services

- parking with desirable traffic flow

- storm water management

- modern entryway and vestibule

- accessibility

- exterior covered area with benches

- Elevator must be added

- One Low Impact Development technique

- Entry and Circulation Desk area

- Children’s Help Desk

- Open area for stacks and computer terminals on both floors

- Public restroom on both floors

- Head Librarian’s office

- Staff workroom/ kitchenette

- Staff restroom

- Two meeting rooms

- Noblesville History room

- Mechanical room

- Books out of direct sunlight

- Green roof with walkway

- Trash receptacle

Building Code and Regulations

-Building Type

This library is public so it will be a type one construction using non - combustible materials. (Reference IBC, chapters 5-6)

-Occupancy

Group A – 3 assembly (Reference IBC, chapter 3)

-Egress

Must meet Egress requirements for an A – 3 building (Reference IBC, chapter 10)

- Number of exits: 2 per story (Reference IBC, Table 1021.1)

- Width of exits: 0.3in stairway per occupant, 0.2in door per occupant (Reference IBC, Section 3412.6.11)

Stairway width=0.3in × 194 people

Stairway width=58.2in

Door width=0.2in × 194 people

Door width=38.8in

-Climate Zone

Zone 5A, cold and moist (Reference Energy Conservation Code)

-Zoning

B: Commercial District (Reference NUDO, Article 7)

- Minimum Lot Size

3 acers (Reference FUDO)

-Maximum Building Hight

35ft (Reference FUDO)

-Roof Live Load

20psf but will be adjusted based on snow load calculations (Reference IBC, chapter 16, section 1607)

-Parking Lot

- Ninety-degree (90º) angle space: Twenty-four-foot (24’) wide parking aisle for one- or two-way traffic, 3ft wide one-way sidewalk, 5ft wide two-way sidewalk (Reference, FUDO)

- Driveway Width must be at least 10ft but no more than 24ft wide.

- Lights shall not excide 35ft tall (Reference, FUDO), constant throughout property

- One space per 3 people (Reference FUDO)

Reading Area = 50sqft * person

Stack Area = 100sqft * person

Meeting Rooms = 15sqft * person

Office Space = 150sqft * person

Max Occupancy= Library Area (sqft)Occupant Load Factor (sqft*person)

Reading Area= 6120sqft50sqft*person

Reading Area= 122 people

Stack Area= 4320sqft100sqft*person

Stack Area= 43 people

Meeting Rooms= 360sqft15sqft*person

Meeting Rooms= 24people

Office Space= 720sqft150sqft*person

Office Space= 5 people

Total Max Occupancy=194 people

Number of Parking Spaces= Max Occupancy3

Number of Parking Spaces= 65 spaces

- 51 to 75 spaces need at least 3 accessible parking spaces (Reference IBC, Table 1106.1)

- Loading berths and areas shall be provided

- Screening shall be a ten-foot (10’) wide landscaped buffer with One evergreen tree planted every eight (8) feet; and one shrub planted every twelve (12) feet; and one deciduous tree planted every fifteen (15) feet along the perimeter of the outdoor sales or outdoor display area (Reference FUDO)

- Each island or peninsula shall be at least one hundred eighty (180) square feet in size. 3. Each island or peninsula shall contain at least one (1) tree per one hundred eighty (180) square feet. (Reference FUDO)

- Minimum thirty-foot (30’) buffer yard (Reference FUDO)

- Designed such that there will be no increase in the peak discharge runoff rate as a result of the proposed development (Reference FUDO)

- 10 to 99 spaces need 5% to be landscaped (Reference FUDO)

- Set back of 8ft from all buildings (Reference FUDO)

Composite Floor Load-Span table

Roof Load

-Live Load

20psf (Reference IBC 1607.11)

-Dead Load

Steel Joist 10psf, Plaster ceiling 10psf

-Snow Load

Ps=0.7CsCeCtIsPg

Ps=0.7(1)(1)(1.1)(1.1)(20psf)

Ps=16.94psf

Ps≥IsPg

16.94 ≥(1.1)(20psf)

16.94 ≥22sf

Ps=22psf

-Total Roof Design Load

62psf

Beam Calculations

Dead Load = Roofing 62psf + mechanicals 3psf

(Reference IBS section 1606)

Live Load = stack room 150bpsf (Reference IBS section 1607.1)

Total Load = 215psf

Fy=50 ksi

E = 29,000 ksi

-Interior Beam

- Diagram

- Calculation

w= Total loads psf × Tributary Width ft

w= 215psf × 24ft

w=5160plf

Ra/b= wL2

Ra/b= (5160plf)(30ft)2

Ra/b= 77400lb

Mmax= wL28

Mmax= (5160lb)(30ft)28

Mmax= 580500lb*ft

Mn ≥ Ma× Ωb

Mn ≥(580500lb*ft)(1.67)

Mn ≥ 969435lb*ft

Mn= Fy × Zx

Zx= MnFy

Zx= 969435lb*ft50000psi × 12ft1in

Zx= 232.66in3

w27X84

Vn ≥ Va × Ωv

Vn ≥(77400lb)(1.5)

Vn ≥ 116100lb

Vn=0.6 × Fy × d × tw

Vn=0.6(50000psi)(26.71in)(0.46in)

Vn=368598lb

368598lb ≥116100lb

∆max= 5wL4384EI

∆max= 5(5160plf × 1in12ft)(30ft × 12in1ft)4384(29000000psi)(2850in4)

∆max= 1.14in

L240 > ∆max

(30ft × 12in1ft)240 > ∆max

1.5in >1.14in

L360 > ∆max

(30ft × 12in1ft)360 > ∆max

1in >1.14in

w27x94

∆max= 5(5160plf × 1in12ft)(30ft × 12in1ft)4384(29000000psi)(3270in4)

∆max= 0.99in

1 >0.99in

Final Design: w30x90 because it is lighter than w27x94 and stiffer, so it still meets all the requirements

-Exterior Beam

- Diagram

- Calculations

w= Total loads psf × Tributary Width ft

w= 215psf × 12ft

w=2580plf

Ra/b= wL2

Ra/b= (2580plf)(30ft)2

Ra/b= 38700lb

Mmax= wL28

Mmax= (2580lb)(30ft)28

Mmax= 290250lb*ft

Mn ≥ Ma× Ωb

Mn ≥(290250lb*ft)(1.67)

Mn ≥ 484717.5lb*ft

Mn= Fy × Zx

Zx= MnFy

Zx= 484717.5lb*ft50000psi × 12ft1in

Zx= 116.33in3

w24x55

Vn ≥ Va × Ωv

Vn ≥(38700lb)(1.5)

Vn ≥ 58050lb

Vn=0.6 × Fy × d × tw

Vn=0.6(50000psi)(23.57in)(0.395in)

Vn=279304.5lb

279304.5lb ≥58050lb

∆max= 5wL4384EI

∆max= 5(2580plf × 1in12ft)(30ft × 12in1ft)4384(29000000psi)(1350in4)

∆max= 1.20in

L240 > ∆max

(30ft × 12in1ft)240 > ∆max

1.5in >1.2in

L360 > ∆max

(30ft × 12in1ft)360 > ∆max

1in >1.2in

w24x68

∆max= 5(2580plf × 1in12ft)(30ft × 12in1ft)4384(29000000psi)(1830in4)

∆max= 0.89in

1in >0.89in

Final Design: w24x68

-Girder on Column line 3

- Diagram

- Calculation

w= Total loads psf × Tributary Width ft

w= 215psf × 30ft

w=6450plf

Ra/b= wL2

Ra/b= (6450plf)(24ft)2

Ra/b= 77400lb

Mmax= wL28

Mmax= (6450lb)(24ft)28

Mmax= 464400lb*ft

Mn ≥ Ma× Ωb

Mn ≥(464400lb*ft)(1.67)

Mn ≥ 775548lb*ft

Mn= Fy × Zx

Zx= MnFy

Zx= 775548lb*ft50000psi × 12ft1in

Zx= 186.13in3

W27x84

Vn ≥ Va × Ωv

Vn ≥(77400lb)(1.5)

Vn ≥ 116100lb

Vn=0.6 × Fy × d × tw

Vn=0.6(50000psi)(26.71in)(0.46in)

Vn=368598lb

368598lb ≥116100 lb

∆max= 5wL4384EI

∆max= 5(6450plf × 1in12ft)(24ft × 12in1ft)4384(29000000psi)(2850in4)

∆max= 0.58in

L240 > ∆max

(24ft × 12in1ft)240 > ∆max

1.2in >0.58in

L360 > ∆max

(24ft × 12in1ft)360 > ∆max

0.8in >0.58in

Final Design: w27x84

-Girder on Column Line 5

- Diagram

- Calculation

w= Total loads psf × Tributary Width ft

w= 215psf × 15ft

w=3225plf

Ra/b= wL2

Ra/b= (3225plf)(24ft)2

Ra/b= 38700lb

Mmax= wL28

Mmax= (3225lb)(24ft)28

Mmax= 232200lb*ft

Mn ≥ Ma× Ωb

Mn ≥(232200lb*ft)(1.67)

Mn ≥ 387774lb*ft

Mn= Fy × Zx

Zx= MnFy

Zx= 387774lb*ft50000psi × 12ft1in

Zx= 93.07in3

W24x55

Vn ≥ Va × Ωv

Vn ≥(38700lb)(1.5)

Vn ≥ 58050lb

Vn=0.6 × Fy × d × tw

Vn=0.6(50000psi)(23.57in)(0.395in)

Vn=279304.5lb

279304.5lb ≥58050lb

∆max= 5wL4384EI

∆max= 5(3225plf × 1in12ft)(24ft × 12in1ft)4384(29000000psi)(1350in4)

∆max= 0.61in

L240 > ∆max

(24ft × 12in1ft)240 > ∆max

1.2in >0.61in

L360 > ∆max

(24ft × 12in1ft)360 > ∆max

0.8in >0.61in

Final Design: w24x55

Spread Footing

qallowable=3500psf

Current Footing: 7’ x 7’ x 2.03’

-Columns on Line 2

Interior Beam End Reaction = 77400lb

Interior Girder End Reaction = 77400lb

Total load = 309600lb

A ≥ Pqallowable

A ≥ 309600lb3500psf

A ≥ 88.46sf

A = 99.47sf

Footing: 7’ x 7’ x 2.03’

-Columns on Line 3

Green Roof = 309600lb

Roof = 62psf × 180sf

Roof = 11160lb

Total load = 320760lb

A ≥ Pqallowable

A ≥ 320760lb3500psf

A ≥ 91.65sf

A = 99.47sf

Footing: 7’ x 7’ x 2.03’

-Columns on Line 4

Green Roof = 309600lb

Roof = 62psf × 720sf

Roof = 44640lb

Total load = 354240lb

A ≥ Pqallowable

A ≥ 354240lb3500psf

A ≥ 101.21sf

A = 99.47sf

New footing = 8’ x 8’ x 2.03’

Wastewater Discharge

- Invert elevation at sewer main is 766.05ft

- 12in Pipe Diameter (1ft)

-Crown Elevation

Crown Elevation = Invert Elevation + Pipe Diameter

Crown Elevation = 766.05ft + 1ft

Crown Elevation = 767.05ft

-Distance from building to main = 123ft

-Slope

Sewer Lateral Slope= inv. El building-Crown EL+ 12ODDistance

Slope= 778ft-767.05ft+(.5 × .5ft) 416ft*100

Slope = 2.6%

minslope= 0.2% (Reference IPC)

Framing

First Floor Plan

Second Floor Plan

Elevation Views

Section View

Landscaping

14 Evergreen 20’

Site Plan

Schedules

No windows (only curtain walls)